## Project: Build a Wooden Birdhouse

This is a beginner-friendly woodworking project that teaches measuring, cutting, drilling, sanding, and finishing.

### Goal

Build a weather-resistant birdhouse that can be mounted on a tree or fence.

### Materials

- 1 untreated pine board (about 15 cm × 120 cm / 6 in × 48 in)

- Wood screws or nails

- Wood glue (optional)

- Sandpaper (80-, 120-, and 220-grit)

- Outdoor, bird-safe paint or wood sealant (optional)

- Small hinges (optional, for a cleaning door)

### Tools

- Tape measure

- Pencil

- Hand saw or jigsaw

- Drill with drill bits

- Screwdriver or drill driver

- Safety glasses and work gloves

### Steps

- Plan the design

  - Sketch the birdhouse.

  - Decide on its dimensions and the size of the entrance hole (this depends on the bird species you want to attract).

- Measure and cut

  - Cut pieces for:

    - Front

    - Back

    - Two sides

    - Floor

    - Two roof panels

- Drill the entrance

  - Drill a circular entrance near the top of the front panel.

  - Drill a few small drainage holes in the floor.

  - Add ventilation holes near the top of the side panels.

- Sand everything

  - Smooth all edges to prevent splinters.

- Assemble

  - Attach the sides to the floor.

  - Add the front and back panels.

  - Install the roof.

  - If using a hinge, make one side or the roof open for cleaning.

- Finish

  - Paint or seal only the outside.

  - Leave the inside untreated.

- Install

  - Mount the birdhouse 2–4 meters (6–13 feet) above the ground in a quiet location.

### Skills You'll Learn

- Reading a plan

- Measuring accurately

- Safe tool use

- Woodworking basics

- Project planning and problem-solving